# Chapelizod Road and Conyngham Road Active Travel Scheme.

# Accessible Description of Drawings.

Chapelizod Road and Conyngham Road English Version

Table of Contents.

1.	Overview.	3

2.	Page Number 1 / Drawing Number 1 – Chapelizod Road.	3

3.	Page Number 2 / Drawing Number 2.	4

4.	Page Number 2 / Drawing Number 3.	5

5.	Page Number 3 / Drawing Number 4.	6

6.	Page Number 3 / Drawing Number 5.	7

7.	Page Number 4 / Drawing Number 6.	7

8.	Page Number 4 / Drawing Number 7.	8

9.	Page Number 5 / Drawing Number 8.	11

10.    Page Number 5 / Drawing Number 9.	11

## Overview.

The Chapelizod Road and Conyngham Road Active Travel Scheme forms part of the Dublin City Cycle Network Plan. The project includes changes to the walking and cycling infrastructure along the route. The scheme design drawings are described in the following document.

All references to “The Project” or “The Scheme” in this document describe the Chapelizod Road and Conyngham Road Active Travel Scheme only.

The project begins at the pedestrian crossing located twenty metres north-west of the Kings Hall entrance at Chapelizod Road. It continues south-east along Chapelizod Road for 2.25 kilometres, past Nativity of the Blessed Virgin Mary church, Liffey Valley Park entrance, Phoenix Park entrances, Saint Brendan’s Gaelic Athletic Association Longmeadows and Garda Boat Club. The scheme includes the junction between Chapelizod Road, South Circular Road and Conyngham Road and continues eastwards along Conyngham Road for seven hundred and fifty metres. The scheme terminates at the junction between Conyngham Road and Chesterfield Avenue, in front of the Phoenix Park entrance. The context is mostly residential with some sports clubs, businesses and parkland. The scheme length is three kilometres, and all drawings show the cycle track in red asphalt.

The drawing pack for the Chapelizod Road and Conyngham Road Active Travel Scheme contains five pages and a total of nine drawings. The first page includes one drawing and the subsequent four pages include two drawings each. The drawings show new walking and cycling infrastructure along the route.

## Page Number One / Drawing Number One – Chapelizod Road.

#### General Overview.

The route orientation runs from north-west to south-east in drawing number one.

Drawing Number one starts on the eastern end of the overall scheme. Starting at the crossing located twenty metres north-west of the Kings Hall estate. The existing crossing will be changed to a toucan crossing, which is four metres wide and ten metres long. The crossing includes a pocket on the northern cycle track for cyclists to access a small, shared space, cross to the southern side and access the new two-way cycle track. The proposed shared space will be located on the existing footpath and will be thirty square metres, Corduroy tactiles will be provided at either end of the shared space. This section of the route passes by the residential developments Springvale Grove and Kings Hall and the Church, Nativity of the Blessed Virgin Mary.

Drawing Number one shows a two-way segregated cycle track 2.5 meters wide on the southern side of the road for a total length of two hundred and fifty meters, the cycle track will be at road level. A kerb height of sixty millimetres is used to segregate the cycle track from the six meters wide road. The existing footpath is 2.4 meters wide on the north side of the road and three meters wide on the south side of the road.

There is no parking at the section of the route.

Segregation between the cycle track and the road is achieved by a concrete buffer approximately 0.5 meters wide and a planted verge with the width approximately two meters.

A yellow box will be placed on the carriageway in front of the Kings Hall entrance to prevent drivers on Chapelizod Road stopping and blocking the entrance and allow drivers exit and from Kings Hall.

There is no Bus stop on this section of the route.

## Page Number Two / Drawing Number Two.

#### General Overview.

Drawing Number two starts sixty meters west of the Chapelizod Road / Upper Glen Road Junction and finishes at two hundred and ninety meters east of the same junction.

The route orientation changes from north-west to south-east in drawing number one to east to west in drawing number two.

The drawing shows a two-way segregated cycle track 2.5 meters wide on the south side of the road for a total length of three hundred and forty meters, the cycle track will be at road level. A sixty millimetres kerb is used to segregate the cycle track from the six meters wide road. The sections of existing cycletrack which is at footpath level on both sides of the road will be removed. This will mean that the footpath width will increase  to three meters.

The existing signalized crossing ten meters west of the entrance to the Phoenix Park at Upper Glen Road, will be upgraded to a toucan crossing where it will be widened from two meters to four meters. Drawing Number two does not include parking along Chapelizod Road, but a loading bay will be provided in front of the Dublin City Council pump station, located one hundred and ninety metres east of the junction on the south side of the road.

Segregation between the cycle track and the road is achieved by a planted verge with the width varying between 1.4 and 2.8 meters.

Each bus stop is described below starting from west to east along the route.

#### Island Bus Stop 2195 (Outbound).

The existing bus stop will be changed to a bus island, and a new shelter will be installed on the new island. The stop is located on the southern side of Chapelizod Road, fifty meters west of the Phoenix Park entrance at Upper Glen Road Junction. At the Bus Stop, the cycle track is taken around the rear of the island adjacent to the footpath, which is three metres wide. The cycle track is ramped up onto the footpath level around the bus stop island. The cycle track is set back three metres from the road to provide an island boarding zone. The island is seventeen meters long and three meters wide.  Zebra-style markings, tactile paving, and a level surface are provided to guide users across. The zebra crossing for pedestrians accessing the bus stop island is 2.5 meters wide.

#### Bus Stop 2247 (Inbound).

The existing in-line bus stop 2247 shelter will be retained. The stop is located on the northern side of Chapelizod Road, fifty meters east of the entrance to the Phoenix Park/ Upper Glen Road Junction. The landing zone is the existing footpath.

## Page Number Three / Drawing Number Three.

#### General Overview.

Drawing Number three starts one hundred fifty-five meters west of the Liffey Valley Park pedestrian entrance and finishes at one hundred sisty-five meters east of the entrance.  The route orientation runs from west to east in drawing number three.

Drawing Number three shows a two-way segregated cycle track 2.5 meters wide on the south side of the road for a total length of three hundred and forty meters, the cycle track will be at road level. A kerb height of sixty millimetres is used to segregate the cycle track from the six meters wide road. The sections of existing cycletrack which is at footpath level on both sides of the road will be removed. This will mean that the footpath width will increase  to 3.1 meters on the northern side and 2.4 meters on the southern side.

Segregation between the cycle track and the road is achieved by a planted verge with the width varying between 1.2 and 1.6 meters.

Each bus stop is described below starting from west to east along the route.

#### Island Bus Stop 2194 (Outbound).

The existing bus stop will be changed to a bus island, and a new shelter will be installed on the footpath. The stop is located on the southern side of Chapelizod Road, one hundred thirty-five meters east of the Liffey Valley Park pedestrian entrance. At the bus stop, the cycle track is taken around the rear of the stop adjacent to the footpath, which is locally 3.6 meters wide. The cycle track is ramped up onto the footpath level around the bus stop and has been narrowed to two metres wide. The cycle track is set back two metres from the road to provide an island boarding zone. This island is thirty meters long and two meters wide. Zebra-style markings, tactile paving, and a level surface are provided to guide users across. The zebra crossing for pedestrians accessing the bus stop island is eight metres wide.

#### Bus Stop 2248 (Inbound).

The existing in-line bus stop 2248 will be relocated eastwards fifteen metres. The stop is located on the northern side of Chapelizod Road, one hundred and sixty meters east of the Liffey Valley Park pedestrian entrance. The landing zone is on the existing footpath.

## Page Number Three / Drawing Number Four.

#### General Overview.

Drawing Number four starts one hundred meters west of the Longmeadows Colaiste Mhuire Sports Ground entrance and finishes at twenty meters east of the Garda Boat Club car exit.

The route orientation runs from west to east in drawing number four.

The drawing shows a two-way segregated cycle track 2.5 meters wide on the south side of the road for a total length of three hundred and fourty meters, the cycle track will be at road level. A kerb height of sixty millimetres is used to segregate the cycle track from the six meters wide road. The sections of existing cycletrack which is at footpath level on both sides of the road will be removed. This will mean that the footpath width will increase to three meters on the northern side and 2.6 meters on the southern side.

This section includes nineteen new parking bays and one disabled parking along Chapelizod Road, these will be at road level, 2.8 meters wide and five meters long. Additionally, a bike parking area ten metres long and 2.8 metres wide will be provided. The twenty new parking bays and the bike parking bay are located on the south side of the road between the road and the cycle track in front of the Longmeadows Colaiste Mhuire Sports Ground entrance.

Segregation between the cycle track and the road is achieved by a planted verge with the width varying between 1.25 meters and 2.5 meters.

There is one bus stop on this section which is described below:

#### Island Bus Stop 2193 (Outbound).

The existing bus stop will be changed to a bus island, and a new shelter will be installed on the new island. The stop is located on the southern side of Chapelizod Road, one hundred meters west of the Garda Boat Club entrance. At the bus stop, the cycle track is taken around the rear of the stop adjacent to the footpath, which is locally 1.8 metres wide. The cycle track is ramped up onto the footpath level around the bus stop. The cycle track is set back three metres from the road to provide an island boarding zone. This island is forty meters long and three meters wide

Zebra-style markings, tactile paving, and a level surface are provided to guide users across. The zebra crossing for pedestrians accessing the bus stop island is three metres wide.

## Page Number Three / Drawing Number Five.

#### General Overview.

Drawing number five starts thirty-five meters west of the Garda Boat Club entrance and finishes at seventy east of the UCD Boat Club entrance.

The route orientation changes from east to west in drawing number four to south-west to north-east in drawing number five.

Drawing five shows a two-way segregated cycle track 2.5 meters wide on the south side of the road for a total length of three hundred and forty meters, the cycle track will be at road level. A kerb height of sixty millimetres is used to segregate the cycle track from the six meters wide road. The sections of existing cycletrack which is at footpath level on both sides of the road will be removed. This will mean that the footpath width will increase to 3.6 meters on the northern side and two meters on the southern side.

This section shows the cycle track connecting to the access to the new bridge, that will lead to the Irish National War Memorial Gardens. The new Bridge is located seventy meters east of the UCD Boat Club entrance. A small, shared space of seventy square metres is proposed on the footpath to facilitate cyclists and pedestrians access to the new bridge. A toucan crossing ten meters long and four meters wide will be implemented five meters east of the new bridge, red tactile paving is placed at either end of the toucan crossing, and corduroy tactiles will be provided at either end of the shared space.

Segregation between the cycle track and the road is achieved by a planted verge with the width of approximately 1.5 meters.

There is one bus stop along this section which is described below:

#### Bus Stop 2249 (Inbound).

The existing in-line bus stop 2249 is located on the northern side of Chapelizod Road, forty meters east of the Garda Boat Club entrance. The landing zone is on the existing footpath.

## Page Number Four / Drawing Number Six.

#### General Overview.

Drawing Number six runs along Chapelizod Road. It starts fifteen metres east of the new bridge that will connect Chapelizod Road and Irish National War Memorial Gardens and finishes fifty meters west of the Khyber Road (Phoenix Park vehicular exit only).

The drawing shows a two-way segregated cycle track 2.5 meters wide on the southern side of the road for a total of three hundred and fifty meters. A kerb height of sixty millimetres is used to segregate the cycle track from the six meters wide road. The sections of existing cycletrack which is at footpath level on both sides of the road will be removed. This will mean that the footpath width will increase  to three meters on the northern side and 2.5 meters on the southern side.

Segregation between the cycle track and the road is achieved by a concrete buffer approximately 0.5 meters wide and a section with a grass verge 1.5 meters wide.

Each bus stop is described below starting from west to east along the route.

#### Island Bus Stop 2192 (Outbound).

The existing bus stop will be changed to a bus island, and a new shelter will be insstalled on the footpath. The stop is located on the southern side of Chapelizod Road, one hundred meters east of the new bridge. At the Island Bus Stop, the cycle track is taken around the rear of the stop adjacent to the footpath, which is locally 3.1 meters wide. The cycle track is ramped up onto the footpath level around the bus stop and has been locally narrowed to two metres. The cycle track is set back two metres from the road to provide an island boarding zone. This island is thirty meters long and two meters wide. Zebra-style markings, tactile paving, and a level surface are provided to guide users across. The zebra crossing for pedestrians accessing the bus stop island is eight meters wide.

#### Bus Stop 2250 (Inbound).

The existing in-line bus stop 2250 will be maintained, it is located on the northern side of Chapelizod Road, one hundred twenty-five meters east of the new bridge. The landing zone is on the existing footpath.

## Page Number Four / Drawing Number Seven.

#### General Overview.

Drawing Number seven runs along Chapelizod Road, passes through the South Circular Road junction, continues eastwards and transitions to Conyngham Road. It starts fifty metres west of the Khyber Road / Phoenix Park exit and finishes seventy-five meters east of the South Circular Road junction.

The drawing shows a two-way segregated cycle track 2.5 meters wide on the southern side of the road for a total of three hundred and fifty meters. A kerb height of sixty millimetres is used to segregate the cycle track from the six meters wide road. The footpath will be 2.5 meters wide on the northern side of the road and 2.3 meters wide on the southern side of the road.

Segregation between the cycle track and the road is achieved by a planted verge and concrete buffer with the width varying between 1.9 and 4.4 meters.  The yellow box will be retained twenty-five meters east of Khyber Road.

A toucan crossing ten meters long and four meters wide will be implemented fifteen meters east of Khyber Road, red tactile paving is placed at either end of the toucan crossing.

The drawing shows ten parking spaces and one disabled bay, located on the southern side of Chapelizod Road thirty-five metres east of the Khyber Road. There are two parking areas, one is forty-five meters long with nine spaces and the second is 15 meters long with one parking space and one disabled parking space.  A two-way cycle track 2.5 meters wide is segregated by concrete buffer 2.4 meters wide between the roadway and the parking bays.

#### Chapelizod Road, South Circular Road and Conyngham Road Junction.

At the Chapelizod Road/South Circular Road/Conyngham Road signalized junction, there will be a two-way cycle crossing the junction. The cycletrack will continue eastwards along Conyngham Road. The cycle track is segregated from the road with a sixty millimetres kerb.

A signalised pedestrian crossing with tactile paving will be implemented on Chapelizod Road, Conyngham Road and South Circular Road. The specific lengths are as follows:

- Chapelizod Road (Western Arm): 14.1 meters

- South Circular Road (Southern Arm): 14 meters

- Conyngham Road (Eastern Arm): 14.6 meters

The cycle track transitions from a two-way cycle track on chapelizod Road, to a one-way cycle track cycle tracks on each side of the South Circular Road, on the east side of the road the width is two meters and on the east side it is 1.5 meters.

The southern arm of the junction has been modified and tightened through the introduction of footpath buildouts and landscape areas.

Southwestern Corner (Chapelizod Road/ South Circular Road):

A new footpath will be provided on the western side of the junction along Chapelizod Road for eighty metres to facilitate pedestrian access.

Due to the difference in level between the new and the existing footpath, the new footpath will have a gradual level drop of five percent from South Circular Road for twenty-five meters westwards. Additionally, a handrail will be installed along the existing kerb.

The drawing shows an existing grass area and three trees at the lower level that will remain, this grass area is twenty-nine meters long and starts on the southwest corner of the junction and continues onto South Circular Road direction.

Corduroy tactiles will be implemented in three locations where there are existing steps:

* Location One: On Chapelizod road, on the existing and the new footpath, fifty-five meters west of the junction.

* Location Two: On Chapelizod road, on the existing and the new footpath, forty-five meters west of the junction.

* Location Three: St the corner of the junction, on the existing footpath, fifty-fire meters west of the junction.

The proposal includes three new grass landscaping areas at the southwest corner of the junction:

* Area One (twenty-six square meters): Located on Chapelizod Road between the road and cycle track, which is twenty-seven meters west of the junction.

* Area Two (ten square meters): Located on Chapelizod Road, south of Area one, between the cycle track and the new footpath, which is twenty-one meters west of the junction.

* Area Three (sixty-nine square meters): Located on South Circular Road between the cycle track and the existing grass area at lower level, which is fifteen meters south of the junction.

The existing temporary bollards for cycle lane segregation along South Circular Road carriageway will be replaced with a buffer 0.5 meters wide on each side of the road.

Southeastern Corner Arm (Conyngham Road/South Circular Road):

Due to the difference in level between the new and the existing footpath, the new footpath will have a gradual level drop from South Circular Road for approximately 9.5 meters eastwards tying in onto the existing layout.

The proposal includes one new grass landscaping area at the southeast corner of the junction. The landscape is placed fifteen meters south of the junction, along South circular Road, between the new footpath and the existing one-way cycle track.

Each bus stop and the South Circular Road junction is described below starting from west to east along the route.

#### Island Bus Stop 2191 (Outbound).

The existing bus stop will be changed to a bus island, and a new shelter will be installed on the new island. The stop is located on the southern side of Chapelizod Road, eighty-five meters east of the Khyber Road. At the Island Bus Stop, the cycle track is taken around the rear of the stop adjacent to the footpath, which is 2.3 metres wide. The cycle track is ramped up onto the footpath level around the bus stop island. This island is twenty-four meters long and five meters wide. Zebra-style markings, tactile paving, and a level surface are provided to guide users across. The zebra crossing for pedestrians accessing the bus stop island. is 2.5 meters wide.

#### Bus Stop 2251 (Inbound).

The existing in-line bus stop 2251 is located on the northern side of Chapelizod Road, forty-five meters east of the Khyber Road. The landing zone is on the existing footpath.

## Page Number Five / Drawing Number Eight.

#### General Overview.

Drawing Number Eight runs along Conyngham Road, it starts fifteen meters west of the Bridge Water Quay entrance and finishes one hundred and twenty-five meters east of the Wellington Apartments car entrance. The sheet shows a two-way segregated cycle track 2.5 meters wide on the southern side of the road for a total of 3three hundred and fifty meters. A kerb height of sixty millimetres is used to segregate the cycle track from the six meters wide road. The footpath on both sides of the road will remain 2.5 meters wide.

Segregation between the cycle track and the road is achieved by a planted verge with the width approximately 1.2 meters.

A yellow box will be retained in front of the Bridge Water Quay entrance, and a new loading bay will be provided one hundred metres west of the Phoenix House Block on the south side of the road, along Conyngham Road.

Each bus stop is described below starting from west to east along the route.

#### Bus Stop 1472 (Inbound).

The existing in-line bus stop 1472 is located on the northern side of Conyngham Road, sixty meters east of the Bridge Water Quay entrance. The landing zone is on the existing footpath.

#### Island Bus Stop 1451 (Outbound).

The existing bus stop will be changed to a bus island, and the existing shelter will be retained on the footpath 2.5 metres wide. The stop is located on the southern side of Conyngham Road, one hundred meters east of the Bridge Water Quay entrance. At the Island Bus Stop, the cycle track is taken around the rear of the stop adjacent to the footpath, which is two metres wide. The cycle track is ramped up onto the footpath level around the bus stop. The island is twenty-eight meters long and two meters wide.  Zebra-style markings, tactile paving, and a level surface are provided to guide users across

There zebra crossing for pedestrians accessing the bus stop island is eight meters wide.

## Page Number Five / Drawing Number Nine.

#### General Overview.

Drawing Number nine runs along Conyngham Road, it starts one hundred and twenty-five meters east of the Wellington Apartments car entrance and finishes at the Conyngham Road and Chesterfield Avenue junction. The sheet shows a two-way segregated cycle track 2.5 meters wide on the southern side of the road for a total of three hundred and fifty meters. A kerb height of sixty millimetres is used to segregate the cycle track from the six meters wide road. The footpath on both sides of the road will remain 2.5 meters wide on the northern side of the road and 3.3 meters wide on the southern side.

Segregation between the cycle track and the road is achieved by a planted verge with the width approximately 1.29 meters.

A yellow box will be retained in front of the Dublin Bus Conyngham Road development. The existing pedestrian crossing located in front of the Phoenix House Block and the toucan crossing located at the Chesterfield Avenue will be retained. A small, shared space of forty-five square metres will be implemented on the southern footpath. Corduroy tactiles will be provided on each side of the shared space.

Each bus stop is described below starting from west to east along the route.

#### Island Bus Stop 1450 (Outbound).

The existing bus stop will be changed to a bus island. The stop is located on the southern side of Conyngham Road, one hundred meters west of the Phoenix House Block. At the Island Bus Stop, the cycle track is taken around the rear of the stop adjacent to the footpath, which is 2.2 metres wide. The cycle track is narrowed to two meters wide and ramped up onto the footpath level around the bus stop. The island is thirty meters long and 1.8 meters wide.  Zebra-style markings, tactile paving, and a level surface are provided to guide users across. The zebra crossing for pedestrians accessing the bus stop island is eight meters wide.

#### Bus Stop 4408 (Inbound).

The existing in-line bus stop 4408 is located on the northern side of Conyngham Road, thirty meters west of the Phoenix House Block. The landing zone is on the existing footpath.

#### Island Bus Stop 1449 (Outbound).

The existing bus stop will be changed to a bus island, and the existing shelter will be retained on the footpath. The stop is located on the southern side of Conyngham Road, fifty-five meters west of the Chesterfield Avenue pedestrian crossing. At the Island Bus Stop, the cycle track is taken around the rear of the stop adjacent to the footpath, which is three metres wide. The cycle track is narrowed to two meters wide and ramped up onto the footpath level around the bus stop. The island is thirty meters long and two meters wide Zebra-style markings, tactile paving, and a level surface are provided to guide users across. The zebra crossing for pedestrians accessing the bus stop island is eight meters wide.

#### Bus Stop 1473 (Inbound).

The existing in-line bus stop 1473 is located on the northern side of Conyngham Road, one hundred meters west of the Chesterfield Avenue pedestrian crossing. The landing zone is on the existing footpath.